# IoT Smart Washroom Hygiene System

## Hardware Setup & Pin-to-Pin Connection Documentation

### 1. System Overview

This document describes the complete hardware setup of the IoT-based Smart Washroom Hygiene System using XIAO ESP32-C6, MQ-136, MQ-137, and CCS811 sensors. It includes pin-to-pin wiring details, operating voltages, power requirements, and electrical considerations.

### 2. Components Used

1. XIAO ESP32-C6 Microcontroller

2. MQ-136 Gas Sensor (H2S detection)

3. MQ-137 Gas Sensor (NH3 detection)

4. CCS811 Air Quality Sensor (VOC & eCO2)

5. TP4056 Li-ion Charging Module

6. Li-ion Battery (Recommended: 2000–2500 mAh for 1-week backup)

### 3. Operating Voltages & Electrical Specifications

| Component | Operating Voltage | Typical Current | Notes |
| --- | --- | --- | --- |
| XIAO ESP32-C6 | 3.3V | 80–120 mA (WiFi active) | Deep sleep ~20 µA |
| MQ-136 | 5V | ~150 mA (heater) | Analog output 0–5V |
| MQ-137 | 5V | ~150 mA (heater) | Analog output 0–5V |
| CCS811 | 3.3V | ~20 mA active | I2C communication |

### 4. Pin-to-Pin Wiring Connections

#### 4.1 MQ-136 Connection

| MQ-136 Pin | Connected To (XIAO ESP32-C6) | Description |
| --- | --- | --- |
| VCC | 5V | Heater power supply |
| GND | GND | Common ground |
| AO | D0 (GPIO0) | Analog gas signal input |
| DO | Not Connected | Digital output not used |

#### 4.2 MQ-137 Connection

| MQ-137 Pin | Connected To (XIAO ESP32-C6) | Description |
| --- | --- | --- |
| VCC | 5V | Heater power supply |
| GND | GND | Common ground |
| AO | D1 (GPIO1) | Analog gas signal input |
| DO | Not Connected | Digital output not used |

#### 4.3 CCS811 Connection (I2C)

| CCS811 Pin | Connected To (XIAO ESP32-C6) | Description |
| --- | --- | --- |
| VCC / 3V3 | 3V3 | Power supply |
| GND | GND | Common ground |
| SDA | D6 (GPIO6) | I2C Data line |
| SCL | D7 (GPIO7) | I2C Clock line |
| WAKE | GND | Keep sensor active |

### 5. Power System Configuration

| TP4056 Pin | Connected To | Purpose |
| --- | --- | --- |
| B+ | Battery + | Battery charging input |
| B- | Battery - | Battery ground |
| OUT+ | 5V (XIAO) | System power output |
| OUT- | GND (XIAO) | System ground |

### 6. Important Electrical Considerations

• All GND connections must be common.

• MQ sensor analog outputs may reach 5V; ensure ADC protection if required.

• Recommended battery capacity for 1-week operation (2-hour interval reading): 2000–2500 mAh.

• Use deep sleep mode between readings for power optimization.

POWER MANAGEMENT

1. XIAO ESP32-C6

| Parameter | Value |
| --- | --- |
| Operating Voltage | 3.3V |
| WiFi Active Current | 80–120 mA |
| CPU Active (no WiFi) | 40–60 mA |
| Light Sleep | ~2–5 mA |
| Deep Sleep | 15–25 µA |

2. MQ-136

| Parameter | Value |
| --- | --- |
| Heater Voltage | 5V |
| Heater Current | ~150 mA |
| Power | ~0.75 W |
| Output Voltage | 0–5V Analog |

3. MQ-137

Same as MQ-136:

| Heater Voltage | 5V |
| Heater Current | ~150 mA |
| Power | ~0.75 W |

Total MQ heater current:

🔥 300 mA @ 5V
🔥 1.5W combined

4. CCS811

| Parameter | Value |
| --- | --- |
| Operating Voltage | 3.3V |
| Active Current | ~15–20 mA |
| Idle Mode | ~1 mA |

BASELINE TOTAL POWER (No Sleep)

Continuous Operation

| Component | Current |
| --- | --- |
| ESP32 WiFi avg | 80 mA |
| MQ-136 | 150 mA |
| MQ-137 | 150 mA |
| CCS811 | 20 mA |

Total ≈ 400 mA

🔋 Weekly Requirement (No Sleep)

400 mA × 168 hours = 67,200 mAh

❌ Requires ~67 Ah battery
❌ Completely impractical

💤 2️⃣ Scenario A — Sleep Every 10 Minutes

Assumptions

- Wake time = 2 minutes

- Sleep time = 8 minutes

- Total cycle = 10 minutes

- 6 cycles per hour

- 144 cycles per day

- 1008 cycles per week

🔹 Active Period Current

~400 mA
Duration = 2 min = 0.033 hr

Energy per cycle:

400 × 0.033 = 13.2 mAh

🔹 Weekly Active Consumption

13.2 × 1008 cycles = 13,305 mAh

🔹 Deep Sleep Consumption

ESP32 deep sleep = 0.02 mA
Sleep time per cycle = 8 min
Total sleep hours per week:

(8/60) × 1008 = 134.4 hours

Energy:

0.02 × 134.4 ≈ 2.7 mAh

Negligible.

✅ Total Weekly Requirement (10-minute interval)

≈ 13,305 mAh
Add 20% overhead:

🔋 ~16,000 mAh required

💤 3️⃣ Scenario B — Sleep Every 2 Hours

Assumptions

- Wake time = 2 minutes

- Sleep time = 118 minutes

- 12 cycles per day

- 84 cycles per week

🔹 Energy Per Cycle

Same as before:

400 mA × 0.033 hr = 13.2 mAh

🔹 Weekly Active Consumption

13.2 × 84 = 1108.8 mAh

🔹 Deep Sleep Energy

Sleep hours per week:

(118/60) × 84 = 165.2 hours

Energy:

0.02 × 165.2 ≈ 3.3 mAh

Negligible.

✅ Total Weekly Requirement (2-hour interval)

≈ 1112 mAh
Add 20% safety:

🔋 ~1350–1500 mAh required

📊 Final Comparison Table

| Scenario | Weekly Consumption | Recommended Battery |
| --- | --- | --- |
| No Sleep | ~67,000 mAh | Not feasible |
| 10-min interval | ~13,300 mAh | 16,000 mAh |
| 2-hour interval | ~1,100 mAh | 1,500–2,000 mAh |

⚠️ Critical Engineering Insight

The system is NOT ESP32 limited.

It is:

🔥 MQ heater limited.

90–95% of power is consumed by MQ sensors.

🧠 Additional Practical Considerations

- MQ sensors require warm-up time.

- Frequent heater cycling may reduce accuracy.

- Cold environments increase heater stabilization time.

- Battery regulator efficiency impacts total capacity.